Ceph对象存储添加osd导致业务io被长时间block问题

问题描述

某项目对象存储对象个数超过1亿3千万，单个bucket接近9千万对象

ceph集群存在1个object omap个数超过5千万，添加osd时会导致业务io长时间block，影响业务正常运行

ceph health detail HEALTH_WARN noout,noscrub,nodeep-scrub flag(s) set; 1 large omap objects OSDMAP_FLAGS noout,noscrub,nodeep-scrub flag(s) set LARGE_OMAP_OBJECTS 1 large omap objects 1 large objects found in pool 'clusterB-zone.rgw.buckets.index' Search the cluster log for 'Large omap object found' for more details.

导致这个告警是由于Ceph默认配置

osd_deep_scrub_large_omap_object_key_threshold= 2000000 # 200万

环境信息

对象存储multisite，2套集群，A集群正常，B集群有个告警

A集群

B集群

问题分析

找到存在large omap object的pgid

准备一个脚本vim large_obj.py

import json import rados import rbd ceph_conf_path = '/etc/ceph/ceph.conf' rados_connect_timeout = 5 class RADOSClient(object): def __init__(self,driver,pool=None): self.driver = driver self.client, self.ioctx = driver._connect_to_rados(pool) def __enter__(self): return self def __exit__(self, type_, value, traceback): self.driver._disconnect_from_rados(self.client, self.ioctx) class RBDDriver(object): def __init__(self,ceph_conf_path,rados_connect_timeout,pool=None): self.ceph_conf_path = ceph_conf_path self.rados_connect_timeout = rados_connect_timeout self.pool = pool def _connect_to_rados(self, pool=None): client = rados.Rados(conffile=self.ceph_conf_path) try: if self.rados_connect_timeout >= 0: client.connect(timeout= self.rados_connect_timeout) else: client.connect() if self.pool == None: ioctx = None else: ioctx = client.open_ioctx(self.pool) return client, ioctx except rados.Error: msg = "Error connecting to ceph cluster." client.shutdown() raise msg def _disconnect_from_rados(self, client, ioctx=None): if ioctx == None: client.shutdown() else: ioctx.close() client.shutdown() class cmd_manager(): def get_large_omap_obj_poolname(self): with RADOSClient(RBDDriver(ceph_conf_path,rados_connect_timeout)) as dr: result = '' cmd = '{"prefix": "health", "detail": "detail", "format": "json"}' result = dr.client.mon_command(cmd,result) if result[0] == 0: res_ = json.loads(result[1]) if res_["checks"]['LARGE_OMAP_OBJECTS']: return res_["checks"]['LARGE_OMAP_OBJECTS']['detail'][0]['message'].split("'")[1] else: return False def get_pg_list_by_pool(self,poolname): with RADOSClient(RBDDriver(ceph_conf_path,rados_connect_timeout)) as dr: result = '' cmd = '{"prefix": "pg ls-by-pool", "poolstr": "' + poolname + '", "format": "json"}' result = dr.client.mon_command(cmd,result) if result[0] == 0: return json.loads(result[1]) else: return False cmd_ = cmd_manager() poolname = cmd_.get_large_omap_obj_poolname() print "Large omap objects poolname = {0}".format(poolname) res = cmd_.get_pg_list_by_pool(poolname) for i in res: if i["stat_sum"]["num_large_omap_objects"] != 0: print "pgid={0} OSDs={1} num_large_omap_objects={2}".format(i["pgid"],i["acting"],i["stat_sum"]["num_large_omap_objects"])

查找large omap所在的pg

python large_obj.py Large omap objects poolname = clusterB-x-zone.rgw.buckets.index pgid=30.91 OSDs=[141, 215, 127] num_large_omap_objects=1

查询pg详情

ceph pg 30.91 query { "state": "active+clean", "peer_info": [ { "peer": "215", "pgid": "30.91", "stats": { "last_deep_scrub_stamp": "2020-10-02 02:28:35.978219", "stat_sum": { "num_bytes": 0, "num_objects": 1, "num_large_omap_objects": 1 }, "up": [ 141, 215, 127 ], "acting": [ 141, 215, 127 ], "blocked_by": [], "up_primary": 141, "acting_primary": 141 } } ] }

找到pg所在的3个osd，查看omap文件总大小，osd上的omap过多导致文件很大19G，正常的osd omap文件夹不超过6G

[root@xxxxx ~]# ssh xxxxx Warning: Permanently added 'xxxxxxx,x.x.x.x' (ECDSA) to the list of known hosts. Last login: Thu May 13 15:51:32 2021 from x.x.x.x [root@xxxxx ~]# du -sh /var/lib/ceph/osd/ceph-215/current/omap/ 16G /var/lib/ceph/osd/ceph-215/current/omap/ [root@xxxxx ~]# logout Connection to xxxxx closed. [root@xxxxx ~]# ssh xxxxx Warning: Permanently added 'xxxxx,x.x.x.x(ECDSA) to the list of known hosts. Last login: Thu May 13 16:55:45 2021 from x.x.x.x [root@xxxxx ~]# du -sh /var/lib/ceph/osd/ceph-127/current/omap/ 18G /var/lib/ceph/osd/ceph-127/current/omap/ [root@xxxxx ~]# logout Connection to xxxxx closed. [root@xxxxx ~]# ssh xxxxx Warning: Permanently added 'xxxxx,x.x.x.x(ECDSA) to the list of known hosts. Last login: Fri May 14 18:57:35 2021 from x.x.x.x [root@xxxxx ~]# du -sh /var/lib/ceph/osd/ceph-141/current/omap/ 19G /var/lib/ceph/osd/ceph-141/current/omap/ [root@xxxxx ~]#

查找ceph 所有节点log搜索Large omap object found，找到一个桶分片omap对象过大，超过5千万条数据，占用空间大约11.5G

2021-01-26 02:43:53.055519 osd.141 osd.141 x.x.x.x:6802/668087 88512 : cluster [WRN] Large omap object found. Object: 30:8905fecf:::.dir.6210e962-feba-4520-8b2e-6e02ed30540c.131681127.2:head Key count: 53957025 Size (bytes): 12445786687

查询可疑对象所在osd

ceph osd map clusterB-zone.rgw.buckets.index .dir.6210e962-feba-4520-8b2e-6e02ed30540c.131681127.2 osdmap e109640 pool 'clusterB-x-zone.rgw.buckets.index' (30) object '.dir.6210e962-feba-4520-8b2e-6e02ed30540c.131681127.2' -> pg 30.f37fa091 (30.91) -> up ([141,215,127], p141) acting ([141,215,127], p141)

对比2个集群，分析该对象是由于之前执行桶分片操作时残留的索引

.dir.6210e962-feba-4520-8b2e-6e02ed30540c.131681127.2，分别在2个集群上找

A集群查询没有该索引对象

B集群查询索引对象存在，但是桶bucket-iep-usc当前并未使用该索引对象

解决办法

删除残留索引对象可临时解决该问题

rados rm .dir.6210e962-feba-4520-8b2e-6e02ed30540c.131681127.2 -p clusterB-x-zone.rgw.buckets.index

操作过程中需要注意：

- 在删除omap数据过程中，由于rocksdb操作时间过长会出现osd down的现象，等删除完osd会自动恢复为up。

- 删除完再次检查osd omap大小，可以看到明显减小，在合理范围内；

- 删完数据集群的告警one large omap objects并不会自动消失，手动执行deep-scrub即可，ceph pg deep-scrub 30.91

- 另外在实施过程中遇到备份数据耗时过长问题，通过命令行大约每秒只能备份100条数据，因此不建议通过命令行备份

mkdir objomap.bak cd objomap.bak poolname="clusterB-x-zone.rgw.buckets.index" objname=".dir.6210e962-feba-4520-8b2e-6e02ed30540c.131681127.2" rados -p $poolname listomapkeys ${objname} > ${objname}.key.omap for id in `cat ${objname}.key.omap`;do rados -p $poolname getomapval ${objname} $id $id;done

使用脚本备份，测试110万条数据1分钟完成，5千万条数据预估45分钟备份完成，参考脚本

#!/usr/bin/env python # -*- coding: UTF-8 -*- import os from rados import (Rados, ReadOpCtx, ObjectNotFound) MAX_SIZE = 100000 def backup_obj_omap(pool_name, object_name): cluster = Rados(conffile='/etc/ceph/ceph.conf') cluster.connect() ioctx = cluster.open_ioctx(pool_name) start_str = "" count = 0 with ReadOpCtx(ioctx) as read_op_inst: while True: iter, _ = ioctx.get_omap_vals(read_op_inst, start_str, "", MAX_SIZE) try: ioctx.operate_read_op(read_op_inst, object_name) except ObjectNotFound: return data = list(iter) if len(data) == 0: break dir_name = str(count) os.makedirs(dir_name) for k, v in data: # print(k, v) f = open(os.path.join(str(count), k), "w") f.write(v) f.close() start_str = k count += 1 if ioctx is None: cluster.shutdown() else: ioctx.close() cluster.shutdown() if __name__ == '__main__': backup_obj_omap('images', 'test')

恢复脚本:参考备份脚本修改

建议

1.修改告警阈值osd_deep_scrub_large_omap_object_key_threshold=200000，避免当omap数据过多时导致业务不可用

由于缺省告警阈值单个object 2million k/v太高，当具有大量omap数据的存储桶索引执行recovery时会阻塞客户端访问该桶内数据，最新代码已降低该值到20万，详见:

https://tracker.ceph.com/issues/40583

该patch已合入到L版v12.2.13

https://github.com/ceph/ceph/pull/29175

2.修改桶配额，限制用户一直往同一个桶内上传过多对象，从业务层进行优化，对象过多的情况考虑分开存放到多个桶内。

参考

https://access.redhat.com/solutions/5703641

https://cloud.tencent.com/developer/article/1400660